Name		: [Your Name]

Class		: M IT-2024 Artificial Intelligence

Subject		: Group Assignment Network Security (Week 9)

Network Segmentation and Firewall Segmentation

- Overview

This project demonstrates a secure network segmentation design implemented using Cisco Packet Tracer. The design is based on the Network Segmentation case study provided in the course material (pages 37–39). The network is segmented into User, DMZ (Server), and External networks, secured using an ASA 5506-X firewall. All configurations were tested and validated through end-to-end connectivity and security checks.

- Network Topology Summary

- Network Segment

| Segment | Network | Purpose |
| --- | --- | --- |
| User LAN | 192.168.100.0/24 | Internal user PCs |
| Inside Transit | 10.10.10.0/24 | Router1 to ASA connection |
| Outside Transit | 20.20.20.0/24 | ASA to Router2 connection |
| DMZ / Server LAN | 172.16.10.0/24 | Application and Database Servers |
| Internet (Simulation) | 8.8.8.0/24 | External network simulation |

- Device Used

- Router-PT (Router1)

- Router-PT (Router2)

- ASA 5506-X Firewall

- Cisco 2960 Switches (User, Server, Internet)

- PC-PT (User PCs)

- Server-PT (Application, Database, Internet Server)

- IP Addressing Plan

- Router1:

  - Fa0/0: 192.168.100.1/24

  - Fa1/0: 10.10.10.2/24

- ASA Firewall:

  - Inside: 10.10.10.1/24

  - Outside: 20.20.20.1/24

  - DMZ: 172.16.10.1/24

- Router2:

  - Fa1/0: 20.20.20.2/24

  - Fa0/0: 8.8.8.1/24

- Servers:

  - DMZ Servers: 172.16.10.11 – 172.16.10.14

  - Internet Server: 8.8.8.8

- Network Device Configuration

- Router1 Configuration

interface fastEthernet0/0
 ip address 192.168.100.1 255.255.255.0
 no shutdown

interface fastEthernet1/0
 ip address 10.10.10.2 255.255.255.0
 no shutdown

ip route 0.0.0.0 0.0.0.0 10.10.10.1

- Router2 Configuration

interface fastEthernet1/0
 ip address 20.20.20.2 255.255.255.0
 no shutdown

interface fastEthernet0/0
 ip address 8.8.8.1 255.255.255.0
 no shutdown

ip route 192.168.100.0 255.255.255.0 20.20.20.1
ip route 10.10.10.0 255.255.255.0 20.20.20.1
ip route 172.16.10.0 255.255.255.0 20.20.20.1

- ASA 5506-X Configuration

interface gigabitEthernet1/1
 nameif inside
 security-level 100
 ip address 10.10.10.1 255.255.255.0

interface gigabitEthernet1/2
 nameif outside
 security-level 0
 ip address 20.20.20.1 255.255.255.0

interface gigabitEthernet1/3
 nameif dmz
 security-level 50
 ip address 172.16.10.1 255.255.255.0

- Testing & Validation

Connectivity tests confirmed full access from the User LAN to internal, DMZ, and Internet resources. Security testing verified that external hosts are unable to access internal or DMZ networks, demonstrating correct firewall enforcement.

- Conclusion

This project successfully implements a secure and functional network segmentation architecture. The use of multiple subnets, an ASA firewall, static routing, and NAT ensures strong separation between internal users, servers, and external networks. The final topology meets all assignment requirements and follows industry best practices.

- Assignment File

https://github.com/rifxonly-tech/week9netsecmentation